Melexis EVK75027

Application Note 1: Hidden Functions

Version 1

BECOM Systems GmbH

Gutheil-Schoder-Gasse 17

1230 Wien

AUSTRIA

office.systems@becom-group.com

http://systems.becom-group.com

Melexis EVK75027 – Application Note 1: Hidden Functions

Template No.: 900-519 Rev A

Publication date: March 22, 2019

Subject to change without notice. Errors excepted.

This document is protected by copyright. All rights reserved. No part of this document may be reproduced or transmitted for any purpose in any form or by any means, electronically or mechanically, without expressly written permission by
BECOM Systems.

Windows is a registered trademark of Microsoft.

Table of Contents

1	General Information	5

1.1	Symbols Used	5

2	Feature Description	6

2.1	Saving registers as Factory Default	6

2.2	Direct access to MLX75027 registers	6

2.3	MLX75027  initialization register map	6

3	Document Revision History	8

A	List of Figures and Table	9

© BECOM Systems GmbH 2019

All Rights Reserved.

The information herein is given to describe certain components and shall not be considered as a guarantee of characteristics.

Terms of delivery and rights of technical change reserved.

We hereby disclaim any warranties, including but not limited to warranties of non-infringement, regarding circuits, descriptions and charts stated herein.

BECOM Systems makes and you receive no warranties or conditions, express, implied, statutory or in any communication with you. BECOM Systems specifically disclaims any implied warranty of merchantability or fitness for a particular purpose.

BECOM Systems takes no liability for any damages and errors causing of the usage of this board. The user of this board is responsible by himself for the functionality of his application. He is allowed to use the board only if he has the qualification. More information is found in the General Terms and Conditions (AGB).

Information

For further information on technology, delivery terms and conditions and prices please contact BECOM Systems www.becom-group.com

# General Information

This guide applies to the Melexis EVK75027 from BECOM Systems and the MLX75027 ToF sensor.

It contains a short description of firmware functionality that is not communicated to customers in the Software User Manual because of its confidentiality.

The document applies to product V0.11.x

## Symbols Used

This guide makes use of a few symbols and conventions:

|  | Warning |
| --- | --- |
|  | Indicates a situation which, if not avoided, could result in minor or moderate injury and/or property damage or damage to the device. |
|  | Caution |
|  | Indicates a situation which, if not avoided, may result in minor damage to the device, in malfunction of the device or in data loss. |
|  | Note |
|  | Notes provide information on special issues related to the device or provide information that will make operation of the device easier. |
|  | Procedures |
|  | A procedure always starts with a headline - The number indicates the step number of a certain procedure you are expected to follow. Steps are numbered sequentially. This sign indicates an expected result of your action. |
|  | References |
|  | This symbol indicates a cross reference to a different chapter of this manual or to an external document. |

# Feature Description

## Saving registers as Factory Default

In order to save the register map as factory default please execute the following steps:

- Register CmdEnablePasswd 0x0022: Write value 0x9C65

- Register CmdExec 0x0033: Write value 0x19D6

## Direct access to MLX75027 registers

The control interface allows direct access to the MLX75027 ’s registers. Register addresses are shifted by 0x8000 to be addressable alongside the control interface registers:

| Register name | MLX75027 Register Address | Control Interface Register Address |
| --- | --- | --- |
| REG_MLX75027_0000 | 0x0000 | 0x8000 |
| REG_MLX75027_0001 | 0x0001 | 0x8001 |
|  | … | … |
| REG_MLX75027_532F | 0x532F | 0xD32F |

Table 1: Register mapping from MLX75027  to Control Interface

Sensor registers can not be saved alongside the camera registers. See chapter 6 for information on how to change sensor register settings permanently.

## MLX75027  initialization register map

At startup, the MLX75027 sensor is initialized with a sequence of register writes read from the camera’s nonvolatile memory.

A default initialization file named init_027_v001.csv (the number value in v001 refers to the file version and might increase in the future) is located at /mnt/settings/.

An override file named override_027.csv can be placed at /mnt/user/ to change values from the default initialization file or to add additional values.

Both files must follow the following file structure and must use Unix/Linux style line endings.

| Line Number | Example Value | Description |
| --- | --- | --- |
| 0 | 0x1234 | Type identifier: 0x1234 for an initialization file, 0x4321 for an override file |
| 1 | 0x1 | File header version |
| 2 | 0x1 | File version (ignored) |
| 3 | This is an example | Custom string for file description |
| 4 | --- | Mandatory separator |
| 5 (...) | 0x1006; 0x08 | Register: (sensor) address; value |

Table 2: Initialization file structure

|  | Caution |
| --- | --- |
|  | Please consult the MLX75027 sensor datasheet prior to changing any sensor registers. Improper use of this feature may result in incorrect data or even permanent damage to the Melexis EVK75027. |

# Document Revision History

| Version | Date | Document Revision |
| --- | --- | --- |
| 1 | 2019 03 13 | Initial version of the document |

Table 3: Document revision history

List of Figures and Table

Figures

No table of figures entries found.

Tables

Table 1: Register mapping from MLX75027  to Control Interface	6

Table 2: Initialization file structure	7

Table 3: Document revision history	8